Importance of N2Hx Reaction Pathways in Modelling of NH3/H2/air Flame Chemistry

Urothiyil Aahil1, Abe Shunnosuke1, Matsuda Dai1, Kitagawa Toshiaki1 ,Ekenechukwu Chijioke Okafor1

1Department of Mechanical Engineering, Kyushu University

7-4-4, Motooka, Nishi-ku, Fukuoka, 819-0395, Japan

1Student

2-16-4, Susenji, Nishi-ku, Fukuoka, 819-0395, Japan

ABSTRACT

A reduced chemical kinetics mechanism capable of modeling the unstretched laminar burning velocity of NH3 and its blends was developed. Existing mechanisms were studied using reaction pathway analysis and sensitivity analysis for burning velocity. The importance of N2Hx related pathways in controlling NH3 and H2 oxidation pathway interactions in NH3/H2/air flames with intermediate H2 concentrations was recognized and described accurately in the new mechanism.

Introduction

Ammonia has gained interest as a carbon free fuel for combustion systems and the blending of ammonia with other faster burning fuel, such as methane and hydrogen, is a method employed to improve the combustion characteristics of ammonia. To develop practical applications with these fuels, numerical simulations that can accurately evaluate the unstretched laminar burning velocity of the flame is required as this property can be used to describe various premixed flame phenomena. Consequently, the development of chemical kinetics mechanisms that can accurately model this property at varied initial conditions are required. Moreover, reaction mechanisms with fewer species and reactions are desirable as large-scale combustion models and computational fluid dynamics simulations that employ bigger chemical kinetics mechanisms involve significantly more computational cost.

Numerous chemical kinetic mechanisms that model NH3/air flames or specific blends of ammonia exist. However, most of these mechanisms were developed, optimized and validated for targeted mixtures, specific set of flame characteristics and select experimental conditions. For instance, a reduced mechanism developed by Okafor et al. [1], a chemical kinetics mechanism derived from a detailed mechanism by Okafor et al. [2] and was optimized for NH3/CH4/air flames. This mechanism satisfactorily predicts SL at equivalence ratios ranging from lean to rich for NH3/air flames [1], [3] and NH3/CH4/air flames [1], [4]. This same reaction mechanism, however, is found to over-predict ignition delay times for NH3/air combustion [5] and SL of NH3/H2/air flames for hydrogen concentrations greater than XH2≈0.4 where XH2 is the molar fraction of the specie H2 in the fuel. The detailed mechanism by Gotama et al. [6], a mechanism based on Han et al.'s mechanism [7], was optimized for NH3/H2/air combustion and then was validated for  XH2≈0.4 at rich, stoichiometric and lean equivalence ratios [6]. Moreover, the mechanisms displayed accurate SL predictions for H2/air flames at various equivalence ratios at atmospheric pressure. Gotama et al. also developed a reduced mechanism (26 species and 119 reactions) based on their detailed mechanism [6]. The mechanism by Nakamura et al. [8] was developed with Miller and Bowman's mechanism [9] as a base to which N2Hx chemistry was added. This led to accurate predictions of NH3, NO and N2O profiles in NH3/air flames. NH3/air flame speed was reasonably predicted by this mechanism at lean and stoichiometric conditions [3] too. Another study found Nakamura Mech's SL predictions of NH3/H2/air flames were satisfactory too, especially in fuel-rich conditions (ϕ=1.1-1.4). However, this mechanism struggles to accurately model NO and N2O profiles in the reaction zones of NH3/air flames. Hence, a demand for a reduced reaction mechanism that can accurately evaluate the unstretched laminar burning velocity of various ammonia blends for a wide range of initial conditions exists.

This study aims to develop a reaction mechanism that can accurately model the SL of NH3/air, NH3/CH4/air and NH3/H2/air flames. The SL, being a flame characteristic that dictates fuel compatability for a wide range of practical applications, is prioritized for this present mechanism. This mechanism should maintain accurate predictions for rich and lean equivalence ratios, elevated pressures and varied fuel compositions.

Numerical Procedure

In this study, the one-dimensional simulations for the unstretched laminar burning velocity, SL, of premixed adiabatic flames were simulated using the Premixed Laminar Flame Speed Calculation Model in ANSYS Chemkin-PRO 2019 R3 and ANSYS Chemkin-PRO 2025 R2. The computation domain was set 10 cm and SL was calculated at the cold boundary of the computation domain. The adaptive grid settings GRAD and CURV were both set to 0.01. The ratio of H2 in the binary fuel, XH2, is defined in (1). XH2 was varied to study how each reaction mechanism's SL predictions are affected by hydrogen addition.

|  | XH2=H2NH3+[H2] | (1) |
| --- | --- | --- |

The software was also used to compute rate of production of species, species concentrations and sensitivity of SL to rate constants of elementary reactions. The reduced chemical kinetics mechanism by Okafor et al. [1], Gotama et al.'s [6] detailed mechanism and Nakamura et al.'s [8] mechanism were used in the simulations. Henceforth, for the purpose of discussion, these mechanisms shall be referred to as Okafor Reduced Mech, Gotama Mech and Nakamura Mech respectively. A reduced chemical kinetic model based on Okafor Reduced Mech. [1] with 43 species and 152 reactions was developed as described in the following section. The developed mechanism will be referred to as Present Mech in this paper.

Development of the Present Mech

Fig. 1 Experimental and numerical laminar burning velocity of NH3/H2/air flames under stoichiometric condition at various hydrogen concentrations.

The new reaction mechanism developed in this paper was based on the Okafor Reduced mechanism. Okafor Reduced Mech was selected because this mechanism has been optimized and validated for SL prediction of NH3 /air and NH3/CH4/air flames. This is true for varying equivalence ratios, pressures and, in the case of NH3/CH4/air flames, varying methane concentrations. This mechanism also models the SL of H2/air flames satisfactorily for most of these conditions. This mechanism performs poorly when it is used to predict SL of NH3/H2/air flames at hydrogen concentrations greater than XH2=0.3 as depicted in Fig. 1, a comparison of numerically simulated values and experimental measurements of burning veloctiy at varying hydrogen concentrations. The mechanism's failure to predict the SL for this specific blend, despite accurately reproducing the individual components of NH3/H2/air flames, motivated the current investigation.

Figure 1's comparison also highlights Gotama Mech and Nakamura Mech's SL predictions agree with experimental measurements of SL at all XH2. Understanding where Gotama Mech and Nakamura Mech differ from Okafor Reduced Mech should provide insights as to where the under-prediction of NH3/H2/air flames stems from. In addition, development of Gotama Mech focused on improving NH3/H2/air flame speed prediction at intermediate hydrogen concentration [6].

To understand how oxidation kinetics of each fuel in the binary fuel interact with each other, reaction pathways for each mechanism were constructed using integrated rate of production (IROP) of major species related to NH3 and H2 oxidation. Each IROP is calculated by integrating the rate of production, ωR,i, of specie i in elementary reaction R along the length of the computational domain (10 cm) using Equation (1). The IROP, hence, characterizes the contribution of each elementary reaction to the combustion of NH3/H2/air fuel in each chemical kinetics model.

|  | 0LωR,i dx | (2) |
| --- | --- | --- |

Figures 2 (a) and (b) are reaction pathways of stoichiometric NH3/H2/air flame of XH2=0.5 computed using Okafor Reduced Mech and Gotama Mech respectively. The thickness of each arrow corresponds to the magnitude of each path’s IROP. Species besides each arrow have numbers that represent the percent contribution each elementary reaction has to the corresponding path. The pathways highlighted the key differences in how the interaction of NH3, and H2 oxidation paths are modelled in each mechanism to inform the development of Present Mech.

Results and Discussion

Comparisons between the pathways of Okafor Reduced Mech and Gotama Mech, Fig. 2 (a) and (b) respectively, highlight interactions between NH3 and H2 oxidation pathways at intermediate H2 concentrations in the binary fuel are modelled differently in the two chemical kinetics models. For instance, reactions in the reaction path NH3↔N2H3 is modelled in Gotama Mech only. The forward reaction, the formation of N2H3, is given by;

|  | NH2+NH=N2H3 | (R1) |
| --- | --- | --- |

and the reverse, the consumption of N2H3, is given by;

|  | 2NH2=N2H3+H | (R2) |
| --- | --- | --- |

Gotama Mech's reaction pathway depicts the involvement of N2H3 in the control of intermediates important in ammonia oxidation pathway.

The reaction pathway NNH→ N2 were distinct between Okafor Reduced Mech and Gotama Mech. Figure 1 shows that Gotama Mech exhibits an IROP approximately 2.2 times larger Okafor Reduced Mech and the pathway is primarily contributed to by the reaction;

|  | NNH=N2+H | (R3) |
| --- | --- | --- |

in Gotama Mech. In Gotama Mech this pathway has become a significant contributor to H radical generation at intermediate hydrogen concentration.

Another reaction pathway, NH2→N2H2 can also be depicted to exhibit increased IROP in Gotama Mech, approximately 3 times larger than in Okafor Reduced Mech. The reaction;

|  | NH2+NH=N2H2+H | (R4) |
| --- | --- | --- |

in the biggest contributor to the pathway. (R4) is a H radical generating reaction while consuming NH2 and NH intermediates. Moreover, the increased N2H2 production further contributes to promote the pathway N2H2→NNH→N2 as the IROP for these pathways in Gotama Mech were approximately 2.8 times and 2.2 times that of Okafor Reduced Mech respectively. In additon, (R1) was the significant reaction in the latter pathway suggesting .

N2Hx species' involvement in NH3 and H2 oxidation pathway interactions as hydrogen concentrations in an NH3/H2/air flame at XH2=0.5 was evident in Gotama Mech. Moreover, the general increase in IROP of N2Hx related reactions, relative to Okafor Reduced Mech's reaction pathway, along with the reactions and reaction pathways discussed in this section, it can be inferred that the N2Hx chemistry has been modelled very differently in the two chemical kinetics models. Hence, to reproduce this phenomenon accurately, Present Mech was developed by adding the N2Hx chemistry from Gotama Mech [6] to Okafor Reduced Mech [1]. The reaction pathway of the Present Mech, hence, behaves similar to Gotama Mech when xH2=0.5 in the binary fuel. Figure \ref{fig-lbv_nh3-h2} shows that, consequently, the Present Mech's SL predictions improve due to this and can now emulate experimental measurements of stoichiometric NH3/H2/air burning velocity accurately across all hydrogen concentrations at atmospheric pressure.

(a) Okafor Reduced Mech 				(b) Gotama Mech

Fig. 2 Reaction pathway of NH3/H2/air flame at XH2=0.5 and equivalence ratio of 1.0 using (a) Okafor Reduced Mech and (b) Gotama Mech.

Figure 2 shows the normalized sensitivity of SL to rate constants of the elementary reactions using the Present Mech. Reaction (R4) is shown to play a significant role in promoting SL in Present Mech while remaining insignificant in Okafor Reduced Mech. In addition, reaction (R1) only significantly inhibits SL in the Present Mech while remaining insignificant Okafor Reduced Mech's sensitiviy analysis. The figure also shows other NHx and N2Hx related reactions being more significant with respect to SL in Present Mech at intermediate hydrogen concentration.

The reactions (R2), (R3) and (R4) involve N2Hx species and control H radical concentrations, an important radical for promoting SL. The increased IROP of these reactions in Present Mech imply an increase in H radical concentration which, correlates with the increased burning veloctiy predictions of this mechanism relative to Okafor Reduced Mech. Moreover, the formation and consumption of N2H3 with (R1) and (R2) respectively control NH and NH2 concentrations. NHx species are important as they interact with H and OH radicals and produce these radicals. (R4) is an example of NHx species' involved in the production of an H radical. The other product of (R4), N2H2, is part of the pathway to form NNH which also produces H radicals via (R3). Moreover, reaction (R4)'s significant positive effect on the flame's burning velocity further supports the theory that N2Hx species and their reaction pathways are important to accurately describe flame with fuel compositions of intermediate hydrogen concentration. Therefore, the Present Mech with a more accurate N2Hx chemistry can predict SL under these conditions accurately.

Validation

The Present Mech was validated with experimental data for SL of NH3/H2/air from the datasets consolidated by Han et al. [10]. These data sets were proven to be consistent with each other and widely used chemical kinetic mechanisms. Present Mech was validated for NH3/CH4/air flames also. Experimental data for NH3/CH4/air SL was compiled from three datasets: Okafor et al. (2019) [1], Han et al. (2019) [11] and Ling et al. (2026) [12]. However, experimental datasets, for both fuel blends, lacked ample SL measurements at elevated pressures, making validation of Present Mech for high pressure combustion difficult.

The results displayed accurate modelling of NH3/H2/air SL at all the equivalence ratios tested for all hydrogen concentrations at atmospheric pressure. At 3 atm of pressure, intermediate H2 concentration (XH2=0.4, 0.6) were validated while, at 5 atm of pressure, only XH2=0.4 could be validated. This suggests that Present Mech is now capable of modelling flame speed of hydrogen enriched ammonia combustion. When comparing NH3/CH4/air predictions to measured SL, the SL predictions are satisfactory. The model can match SL predictions to Okafor et al. (2019) [1] for all the its data points across elevated pressures, increasing XCH4 and different equivalence ratios. This implies that the underlying Okafor Reduced Mech optimizations for NH3/CH4/air flame chemistry in Present Mech is unaffected.

References

E. C. Okafor, Y. Naito, S. Colson, A. Ichikawa, T. Kudo, A. Hayakawa, and H. Kobayashi, Combustion and Flame, 204, (2019), 162–175.

E. C. Okafor, Y. Naito, S. Colson, A. Ichikawa, T. Kudo, A. Hayakawa, and H. Kobayashi, Combustion and Flame, 187, (2018), 185–198.

B. Mei, X. Zhang, S. Ma, M. Cui, H. Guo, Z. Cao, and Y. Li, Combustion and Flame, 210, (2019), 236–246.

W. S. Chai, Y. Bao, P. Jin, G. Tang, and L. Zhou, Renewable and Sustainable Energy Reviews, 147, (2021), 111254.

R. Li, A. A. Konnov, G. He, F. Qin, and D. Zhang, Fuel, 257, (2019), 116059.

G. J. Gotama, A. Hayakawa, E. C. Okafor, R. Kanoshima, M. Hayashi, T. Kudo, and H. Kobayashi, Combustion and Flame, 236, (2022), 111753.

X. Han, Z. Wang, Y. He, Y. Zhu, and K. Cen, Combustion and Flame, 213, (2020), 1–13.

H. Nakamura, S. Hasegawa, and T. Tezuka, Combustion and Flame, 185, (2017), 16–27.

J. A. Miller and C. T. Bowman, Progress in Energy and Combustion Science, 15, (1989), 287–338.

H. Nakamura and S. Hasegawa, Proceedings of the Combustion Institute, 36, (2017), 4217–4226.

J. Otomo, M. Koshi, T. Mitsumori, H. Iwasaki, and K. Yamada, International Journal of Hydrogen Energy, 43, (2018), 3004–3014.

Abe, S., Kitagawa, T., and Okafor E. C., Proceedings of the 35th Internal Combustion Engine Symposium [No. 24-51], (2024).

X. Han, K. Wan, and T. Nagy, Combustion and Flame, 286, (2026), 114855.

X. Han, Z. Wang, M. Costa, Z. Sun, Y. He, and K. Cen, Combustion and Flame, 206, (2019), 214–226.

Z. Ling, J. Lin, J. Li, X. Han, M. Liu, D. Yuan, and Z. Wang, Journal of the Energy Institute, 126, (2026), 102503.