Importance of N2Hx Reaction Pathways in Modelling of NH3/H2/air Flame Chemistry

Urothiyil Aahil1, Abe Shunnosuke1, Matsuda Dai1, Kitagawa Toshiaki1, Ekenechukwu Chijioke Okafor1

1Department of Mechanical Engineering, Kyushu University

7-4-4, Motooka, Nishi-ku, Fukuoka, 819-0395, Japan

1Student

2-16-4, Susenji, Nishi-ku, Fukuoka, 819-0395, Japan

ABSTRACT

A reduced kinetic mechanism was optimized for NH3, NH3/CH4, and NH3/H2 flames to support efficient CFD modeling. 1D flame analyses showed that the under-prediction of burning velocity in mixtures with high-hydrogen blends by the original mechanism stem from insufficient H-atom generation via NH2+NH⇌N2H2+H and NNH⇌N2+H. Updating N2Hx pathways resolved this issue, yielding accurate burning velocity predictions across all target blends.

Introduction

Ammonia (NH3) is a promising carbon-free fuel, frequently blended with methane (CH4) or hydrogen (H2) to enhance reactivity in combustion systems. Combustor development for these blends relies on numerical simulations to accurately predict key phenomena, including laminar burning velocity, ignition dynamics, and pollutant emissions. Because large-scale or three-dimensional modeling using detailed kinetics incurs substantial memory and computation time costs, reliable reduced mechanisms, containing fewer species and reactions, are essential.

Numerous chemical kinetic mechanisms have been developed to model ammonia combustion; however, most are optimized for specific fuel blends and narrow operating conditions. For example, the reduced mechanism by Okafor et al. [1] was designed for NH3/air and NH3/CH4/air flames. While it effectively predicts unstretched laminar burning velocity (SL) across lean-to-rich equivalence ratios for these mixtures [1–3], it significantly underpredicts SL for flames when H2 volume fractions exceed 0.3. Conversely, the detailed and reduced mechanisms by Gotama et al. [4] were tailored for NH3/H2/air combustion, accurately predicting SL for high-hydrogen blends and pure H2/air flames at atmospheric pressure [4]. Similarly, the mechanism by Nakamura et al. [5] captures SL well for fuel-rich NH3/H2/air flames (ϕ=1.1-1.4) [5]. Consequently, there remains a critical need for a single reduced mechanism capable of accurately evaluating the unstretched laminar burning velocity across a wide spectrum of ammonia blends and operating conditions

Okafor et al.’s reduced mechanism [1] under-prediction of SL for NH3/H2/air flames could lie in the complex cross-coupling between ammonia and hydrogen reaction pathways. Because this reduced mechanism accurately models NH3/air, NH3/CH4/air and H2/air flames on their own, its failure at higher hydrogen fractions (XH2>0.3) points to missing or miscalibrated kinetic interactions specific to NH3/H2 co-combustion. At elevated hydrogen levels, the surge in key radical pools (H, O, and OH) alters the primary oxidation routes and radical consumption rates that dictate flame speed [4,5]. These limitations suggest that the mechanism's reaction network may need to be re-examined specifically under conditions where H2 concentration in the fuel mixture is significant.

The primary objective of this work is to develop a robust reduced chemical kinetic mechanism capable of accurately modelling the combustion of NH3/air, NH3/CH4/air, and NH3/H2/air fuel blends across a wide range of operating conditions, building directly upon the reduced model of Okafor et al. [1]. To achieve this, the underlying reaction pathways of NH3/H2/air flames were analysed to elucidate key kinetic interactions between ammonia and hydrogen chemistries that were underrepresented in the original mechanism.

Numerical Procedure

In this study, one-dimensional simulations for the unstretched laminar burning velocity, SL, of premixed adiabatic flames were simulated using the Premixed Laminar Flame Speed Calculation Model in ANSYS Chemkin-PRO. The computation domain was set 10 cm and SL was calculated at the cold boundary of the computation domain. The adaptive grid settings GRAD and CURV were both set to 0.01. The ratio of H2 in the binary fuel, XH2, is defined in Equation (1). XH2 was varied, while atmospheric pressure and stoichiometric equivalence ratio (ϕ=1.0) were maintained, to study how each reaction mechanism's SL predictions are affected by hydrogen addition.

|  | XH2=H2NH3+[H2] | (1) |
| --- | --- | --- |

The software was also used to compute rate of production of species, species concentrations and sensitivity of SL to rate constants of elementary reactions. The reduced chemical kinetics mechanism by Okafor et al. [1] and Gotama et al.'s [4] detailed mechanism was used in the simulations. Henceforth, for the purpose of discussion, these mechanisms shall be referred to as Okafor Reduced Mech and Gotama Mech respectively. A reduced chemical kinetic model, based on Okafor Reduced Mech [1], with 43 species and 152 reactions was developed as described in the following section. The developed mechanism will be referred to as Present Mech in this paper.

Development of the Present Mech

Fig. 1 Experimental and numerical laminar burning velocity of NH3/H2/air flames under stoichiometric condition at various hydrogen concentrations.

The new reaction mechanism developed in this paper was based on the Okafor Reduced mechanism. Okafor Reduced Mech was selected because this mechanism has been optimized and validated for SL prediction of NH3 /air and NH3/CH4/air flames. This is true for varying equivalence ratios, pressures and, in the case of NH3/CH4/air flames, varying methane concentrations. This mechanism also models the SL of H2/air flames satisfactorily for most of these conditions. However, it significantly under-predicts SL of NH3/H2/air flames with hydrogen fractions larger than 0.3 as shown in Fig. 1, hence motivating the need to optimize it for NH3/H2/air flames.

Figure 1 highlights Gotama Mech's SL predictions agree with experimental measurements of SL across all XH2. Understanding where Gotama Mech differs from Okafor Reduced Mech should provide insights as to where the under-prediction of NH3/H2/air flames stems from. In addition, development of Gotama Mech focused on improving NH3/H2/air flame speed prediction at intermediate hydrogen concentration [4].

To understand how oxidation kinetics of each fuel in the binary fuel interact with each other, reaction pathways for each mechanism were constructed using integrated rate of production (IROP) of major species related to NH3 and H2 oxidation. Each IROP is calculated by integrating the rate of production, ωR,i, of specie i in elementary reaction R along the length of the computational domain (10 cm) using Equation (2). The IROP, hence, characterizes the contribution of each elementary reaction to the combustion of NH3/H2/air fuel in each chemical kinetics model.

|  | 0LωR,i dx | (2) |
| --- | --- | --- |

Figures 2 (a) and (b) are reaction pathways of stoichiometric NH3/H2/air flame of XH2=0.5 computed using Okafor Reduced Mech and Gotama Mech respectively. The thickness of each arrow corresponds to the magnitude of each path’s IROP. Species besides each arrow have numbers that represent the percent contribution each elementary reaction has to the corresponding path. The pathways highlighted the key differences in how the interaction of NH3, and H2 oxidation paths are modelled in each mechanism to inform the development of Present Mech.

Normalized sensitivity of SL to rate constants of elementary reactions was subsequently analysed to determine the direct dependence of the laminar burning velocity (SL) on individual elementary reactions. By combining these sensitivity trends with IROP analyses, the dominant reactions and critical cross-coupling pathways dictating flame speed were successfully identified. Based on these insights, targeted modifications made to Okafor Mech to model the under-represented pathways for NH3/H2/air flames at higher hydrogen fractions and develop the Present Mech.

Present Mech then was validated using experimental flame speed measurements from literature for NH3/H2/air, NH3/CH4/air flames across lean (ϕ=0.7), stoichiometric and rich (ϕ=1.4) equivalence ratios. Present Mech’s performance at elevated pressures were evaluated too. These validation steps ensure that the developed mechanism meets the aim of this study and is compatible with various ammonia fuel blends at various operating conditions.

Results and Discussion

Comparisons between the pathways of Okafor Reduced Mech and Gotama Mech, Fig. 2 (a) and (b) respectively, highlight interactions between NH3 and H2 oxidation pathways at intermediate H2 concentrations in the binary fuel are modelled differently in the two chemical kinetics models. For instance, reactions in the reaction path NH2↔N2H3 is modelled in Gotama Mech only. The forward reaction, the formation of N2H3, is given by;

|  | NH2+NH=N2H3 | (R1) |
| --- | --- | --- |

and the reverse, the consumption of N2H3, is given by;

|  | 2NH2=N2H3+H | (R2) |
| --- | --- | --- |

Gotama Mech's reaction pathway depicts the involvement of N2H3 in the control of intermediates important in ammonia oxidation pathway.

The reaction pathway NNH→ N2 are distinct between Okafor Reduced Mech and Gotama Mech. Figure 2 shows that Gotama Mech exhibits an IROP approximately 2.2 times larger Okafor Reduced Mech and the pathway is primarily contributed to by the reaction;

|  | NNH=N2+H | (R3) |
| --- | --- | --- |

in Gotama Mech, which contributes significantly to H radical generation at intermediate hydrogen concentration.

Another reaction pathway, NH2→N2H2 can also be depicted to exhibit increased IROP in Gotama Mech, approximately 3 times larger than in Okafor Reduced Mech. The reaction;

|  | NH2+NH=N2H2+H | (R4) |
| --- | --- | --- |

in the biggest contributor to the pathway. (R4) is a H radical generating reaction while consuming NH2 and NH intermediates. Moreover, the increased N2H2 production further contributes to promoting the pathways N2H2→NNH→N2 as the IROP for these pathways in Gotama

(a) Okafor Reduced Mech 				(b) Gotama Mech

Fig. 2 Reaction pathway of NH3/H2/air flame at XH2=0.5 and equivalence ratio of 1.0 using (a) Okafor Reduced Mech and (b) Gotama Mech.

Mech were approximately 2.8 times and 2.2 times that of Okafor Reduced Mech respectively. In addition, (R1) is the significant reaction in the latter pathway suggesting.

Reactions (R1) and (R2) were added to Present Mech as these reactions were established to be important for controlling NH concentration. These reactions’ roles in the combustion of NH3/H2/air flames with intermediate hydrogen concentrations were also confirmed by Gotama et al. [4]. The rate constants of (R3) was updated by choosing rate constants published by Pagsberg et al. [6]. This enhanced the production of H atoms from the NNH→N2 pathway. Glarborg et al. [7] corroborates the dominance of reaction (R3) in this pathway and confirms the significant role (R3) plays in H radical generation. Rate constants for (R4) were updated with the rate constants published by Klippenstein et al. [8]. Otomo et al. also recognizes the influence of reaction (R4) on the SL of NH3/H2/air flames.

Normalized sensitivity of SL to rate constants of the elementary reactions using the Present Mech and Okafor Mech highlighted reaction (R4) playing a significant role in promoting SL in Present Mech while remaining insignificant in Okafor Reduced Mech. In addition, reaction (R1) only significantly inhibits SL in the Present Mech while remaining insignificant for Okafor Reduced Mech. This analysis also showed other NHx and N2Hx related reactions being more significant with respect to SL in Present Mech at intermediate hydrogen concentration.

The reactions (R2), (R3) and (R4) involve N2Hx species and control H radical concentrations, an important radical for promoting SL. The increased IROP of these reactions in Present Mech implies an increase in H radical concentration, which correlates with the increased burning velocity predictions of this mechanism relative to Okafor Reduced Mech. Moreover, the formation and consumption of N2H3 with (R1) and (R2) respectively control NH and NH2 concentrations. NHx species are important as they interact with H and OH radicals and produce these radicals. (R4) is an example of NHx species' involvement in the production of an H radical. The other product of (R4), N2H2, is part of the pathway to form NNH which also produces H radicals via (R3). Moreover, reaction (R4)'s significant positive effect on the burning velocity further supports the theory that N2Hx species and their reaction pathways are important to accurately describe flame with fuel compositions of intermediate hydrogen concentration. Therefore, the Present Mech with a more accurate N2Hx chemistry can predict SL under these conditions accurately.

Validation

The Present Mech was validated with experimental data for SL of NH3/H2/air from the datasets consolidated by Han et al. [9]. These data sets were proven to be consistent with each other and widely used chemical kinetic mechanisms. Present Mech was validated for NH3/CH4/air flames also. Experimental data for NH3/CH4/air SL was compiled from three datasets: Okafor et al. (2019) [1], Han et al. (2019) [10] and Ling et al. (2026) [11]. However, experimental datasets, for both fuel blends, lacked ample SL measurements at elevated pressures, making validation of Present Mech for high pressure combustion difficult.

Figure 3 compares NH3/CH4/air predictions by the Present Mech to experimentally measured SL from literature at stoichiometric equivalence ratio and atmospheric pressure to depict that the SL predictions are satisfactory. This shows that the present optimization of the N2Hx sub-mechanism does not undermine the fidelity of the mechanism in modelling NH3/CH4/air blends, which the original mechanism was optimized for.

Fig. 5 Comparison of measured and simulated laminar burning velocity of NH3/H2/air flames under lean condition (ϕ=1.4) and 1 atm of pressure.

Figure 4 and 5 shows that the Present Mech satisfactorily models the laminar burning velocity of NH3/H2/air at lean and rich equivalence ratios respectively. The new reduced model was also validated with measurements from Wang et al. [12] at 3

Fig. 4 Comparison of measured and simulated laminar burning velocity of NH3/H2/air flames under lean condition (ϕ=0.7) and 1 atm of pressure.

Fig. 3 Comparison of measured and simulated laminar burning velocity of NH3/CH4/air flames under stoichiometric condition and 1 atm of pressure.

atm and intermediate H2 concentrations (XH2=0.4, 0.6) and Gotama et al. [4] at 5 atm and XH2=0.4. This suggests that Present Mech is now capable of modelling flame speed of hydrogen enriched ammonia combustion.

Conclusion

In this study, a reduced chemical kinetics model was optimized for NH3/air, NH3/CH4/air and NH3/H2/air flames, based on an existing chemical kinetics model developed for NH3/air and NH3/CH4/air flames. A comparative chemical kinetics analysis of the original reduced mechanism to detailed reaction mechanisms in literature showed that the significant underprediction of the laminar burning velocity of the NH3/H2/air flames at intermediate hydrogen volume fractions was due to inaccurate modelling of H atom production from the N2Hx sub-mechanism. By incorporating elementary reactions and optimizing key rate constants within the N2Hx sub-mechanism, NH3/H2/air laminar burning velocity predictions were made more accurate without compromising the mechanism's fidelity for NH3/air and NH3/CH4/air flames. This study therefore shows the unique importance of the N2Hx sub-mechanism in modelling the flame speeds of NH3/H2/air flames.

Acknowledgement

This work was supported by JSPS KAKENHI Grant Number 23K13263.

References

[1]	E.C. Okafor, Naito, S. Colson, A. Ichikawa, T. Kudo, A. Hayakawa, H. Kobayashi, Combust. Flame 204 (2019) 162–175.

[2]	B. Mei, X. Zhang, S. Ma, M. Cui, H. Guo, Z. Cao, Y. Li, Combust. Flame 210 (2019) 236–246.

[3]	W.S. Chai, Y. Bao, P. Jin, G. Tang, L. Zhou, Renew. Sustain. Energy Rev. 147 (2021) 111254.

[4]   Gotama, A. Hayakawa, E.C. Okafor, R. Kanoshima, M. Hayashi, T. Kudo, H. Kobayashi, Combust. Flame 236 (2022) 111753.

[5]	H. Nakamura, S. Hasegawa, T. Tezuka, Combust. Flame 185 (2017) 16–27.

[6]	P.B. Pagsberg, J. Eriksen, H.C. Christensen, J. Phys. Chem. 83 (1979) 582–590.

[7]	P. Glarborg, J.A. Miller, B. Ruscic, S.J. Klippenstein, Prog. Energy Combust. Sci. 67 (2018) 31–68.

[8]	S.J. Klippenstein, L.B. Harding, B. Ruscic, R. Sivaramakrishnan, N.K. Srinivasan, M.-C. Su, J.V. Michael, J. Phys. Chem. A 113 (2009) 10241–10259.

[9]	X. Han, K. Wan, T. Nagy, Combust. Flame 286 (2026) 114855.

[10]	X. Han, Z. Wang, M. Costa, Z. Sun, Y. He, K. Cen, Combust. Flame 206 (2019) 214–226.

[11]	Z. Ling, J. Lin, J. Li, X. Han, M. Liu, D. Yuan, Z. Wang, J. Energy Inst. 126 (2026) 102503.

[12]	S. Wang, Z. Wang, A.M. Elbaz, X. Han, Y. He, M. Costa, A.A. Konnov, W.L. Roberts, Combust. Flame 221 (2020) 270–287.