# Importance of N2Hx Reaction Pathways in Modelling of NH3/H2/Air Flame Chemistry

Aahil Ashraf Urothiyil​1, Abe Shunnosuke​2, Matsuda Dai​2, Kitagawa Toshiaki​2, Ekenechukwu Chijioke Okafor​2  
​1Department of Mechanical Engineering, Kyushu University
7-4-4, Motooka, Nishi-ku, Fukuoka, 819–0395, Japan
​1Student
Street, City, State, Zip-code, Country

# Introduction

Replacing fossil fuels as the primary source of energy is being increasingly more important as the environmental impact from carbon emissions, such as global warming, continue to intensify. Ammonia and ammonia-fuel blends are potential alternative fuels to replace traditional fossil fuels. Ammonia is an excellent hydrogen carrier, another alternative fuel, and the combustion of ammonia produces no carbon products. Moreover, the production, storage and transportation of ammonia is more practical relative to other carbon free fuels such as hydrogen. This along with the existing infrastructure for producing and transporting ammonia are strong reasons for promoting ammonia’s use as a fuel.

However, ammonia combustion poses certain challenges in various applications — ammonia has a slower LBV, smaller lower heating value and narrower flamability limit than fuels such as methane and hydrogen. These demerits need to be overcome for ammonia to be functionally applicable in combustion systems. With regards to emissions, although no carbon dioxide is produced from ammonia combustion, fuel NOx production becomes significant. Understanding NOx production pathways is required to control these emissions.

Improving the combustion properties of ammonia can be achieved by mixing more combustible fuels, like methane and hydrogen, to ammonia. Ideally, such fuel blends should be able to exhibit combustion properties similar to that of traditional fuels while significantly reducing carbon emissions from combustion. Adding fuels with higher LBV than ammonia’s, like methane and hydrogen, can improve the LBV and other properties of the flame. Meanwhile, carbon emissions can be kept to a minimum. Therefore, finding the exact ratios of fuels in the ammonia blends at which these flames properties align with system requirement is neccessary and is possible with numerical simulations with reaction mechanisms.

(*) combine pros and cons of ammonia into one para

The aim of this study is to develop a reaction mechanism to accurately model flames for ammonia and ammonia blends, namely NH3/H2/Air and CH4/NH3/Air flames. This mechanism prioritizes LBV, a critical flame characteristic that dictates fuel compatability for a wide range of practical applications. Most existing reaction mechanisms from literature are only able to reproduce some fuels’ LBV accurately under select pressure, initial temperature and equivalence ratios. Okafor reduced mechanism, for example, is designed to predict CH4/NH3/Air’s LBV. This mechanism also exhibits high reproducibility for LBVs of NH3/Air flames and H2/Air flames however underpredicts LBV of NH3/H2/Air flames. In this study, the reason for the underprediction was studied and a mechanism was developed around the findings.

# Developing Reaction Mechanism

The new reaction mechanism developed in this paper is based on the Okafor reduced mechanism. To study how ammonia and hydrogen interact and affect LBV, Otomo mech, Nakamura mech and Gotama mech were studied. The figure below [Figure lbv nh3/h2] comparing the different numerical simultions with the mentioned reaction mechanisms to experimental data. For NH3/H2/Air flames, LBV predictions by Okafor reduced mech are accurate for smaller hydrogen concentrations but underpredict otherwise. On the other hand other mechanism are able to reproduce this flame property across various hydrogen concentrations.

## Methodology

Numerical simulations for this study is conducted using ANSYS Chemkin-PRO 2019 R3 and ANSYS Chemkin 2025 R2. The "Premixed Laminar Flame Speed Calculation" model was used and the initial conditions kept constant for all the simulations were the pressure at 1.0 atm, temperature at 298.0 K, and equivalence ratio at 1.0. GRAD and CURV parameters were set to 0.01, the computational domain was 10 cm, multicomponent transport was enabled and thermal diffusion was enabled.

From the numerical simulations, integrated rate of production (IROP) and sensitivity of each elementary reaction was evaluated. With the IROP, reaction pathways can be created for each mechanism. Utilizing the sensitivity analyses and reaction pathways, the reactions impacting LBV are highlighted by recognising the differences between Okafor reduced mech and the other mechanisms.

## Findings from Comparing Mechanisms

[Figures reaction pathways]

Comparing Okafor reduced mech to Nakamura mech, few key pathways depict LBV prediction disparity between the mechanisms. NH→N→N2 and HNO→NO→N2 reaction pathway has higher IROP in Nakamura mech. This is observed because of the increase in H radicals as hydrogen is introduced into the fuel blend proliferating NH→N and HNO→NO reactions. The former is primarily caused by the reaction N+NO↔N2+O and the latter by HNO+H↔NO+H2. In other words, Nakamura mech utilizes pathways involving NO to use the excess H radicals. Hence, formation of other radicals, OH and O, are promoted leading to improved LBV predictions.

Next, Gotama mech was studied. Firstly, Gotama mech described NH2↔N2H3 reaction pathway, a pathway neglected in Okafor reduced mech. The reactions NH2+NH↔N2H3 and 2NH2→N2H3+H contribute the most in this reaction pathway and provide another pathway to produce H radicals. The underprediction of the LBV observed in [Fig 1] may be attributed to the absence of this pathway in Okafor reduced mech. Furthermore, NNH↔N2+H reaction is more significant in Gotama mech, another H radical producing elementary reaction. Another H radical reaction that was under-represented in Okafor mech was NH3+NH↔N2H2+H. This reaction also plays a role in NH3 consumption pathway, hence the under-representation negatively affects the LBV prediction.

## Changes in the New Mechanism

The key takeaway from the comparisons made with mechanisms that can accurately predict LBV is that reaction pathways that utilize the additional H2, and the resulting H radicals, and further decompose NH3 are described in detail in the mechanism. Reaction pathways involving NO and N2Hx was incorporated into Okafor reduced mech to develop the new mechanism by adding elementary reactions or replacing the Arrhenius’s function constants of exising reactions with constants from other mechanisms.

[Figure new mech lbv]

The figure above plots how the changes to Okafor reduced mech improved the LBV prediction for NH3/H2/Air flames at varying hydrogen concentrations. As seen above, the new mechanism’s predictions align closely with experimental LBV data from literature. This reinforces the findings from reaction pathway analyses about how important the missing and under-represented reaction pathways, namely the N2Hx pathways, are in reproducing NH3/H2/Air flames.

# Validation

# Concluding Remarks

The due date of the paper submission is July 21, 2026.

99

E. C. Okafor, Y. Naito, S. Colson, A. Ichikawa, T. Kudo, A. Hayakawa, and H. Kobayashi, Combustion and Flame, 204, (2019), 162–175.

A. Valera-Medina, F. Amer-Hatem, A. K. Azad, I. C. Dedoussi, M. De Joannon, R. X. Fernandes, P. Glarborg, H. Hashemi, X. He, S. Mashruk, J. McGowan, C. Mounaim-Rouselle, A. Ortiz-Prado, A. Ortiz-Valera, I. Rossetti, B. Shu, M. Yehia, H. Xiao, and M. Costa, Energy & Fuels, 35, (2021), 6964–7029.

M. Alnajideen, H. Shi, W. Northrop, D. Emberson, S. Kane, P. Czyzewski, M. Alnaeli, S. Mashruk, K. Rouwenhorst, C. Yu, S. Eckart, and A. Valera-Medina, Carbon Neutrality, 3, (2024), 13.

M. El-Adawy, M. A. Nemitallah, and A. Abdelhafez, Fuel, 364, (2024), 131090.

99

Corresponding author: Aahil Urothiyil

E-mail address: urothiyil.aahil.412@s.kyushu-u.ac.jp